# TANGO

Output Transformers • Power Transformers • Chokes • Input/Driver Transformers

English Translation of Japanese Product Catalog

TANGO (by Hirata Electric Manufacturing Co., Ltd.) produces high-quality audio transformers for vacuum tube amplifiers. The X and XE series represent their flagship output transformers, featuring Orient Hi-B cut-core construction for excellent magnetic properties, wide bandwidth, low distortion, and high-grade performance.

This catalog covers push-pull and single-ended output transformers, universal types, power transformers, filter chokes, and specialized driver/input transformers. All models are designed for audiophile-grade tube amplifiers using classic tubes such as 300B, 2A3, EL34, KT-88, 6550, 6L6GC, and many others.

# OUTPUT TRANSFORMERS — X / XE Series (Orient Hi-B Cut Core)

## Push-Pull Types — Secondary: 4Ω, 8Ω, 16Ω

| Model | Power (W) | Primary Zp (Ω) | Response (-3dB) | Primary L (H) Min/Max | DC Current (mA) Total/Unbal. | Loss @16Ω (dB) | Recommended Tubes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X-8P | 120 @30Hz | 8K | 4Hz~60kHz | 220 / 970 | 320 / 6 | 0.16 | EL-156, KT-66 |
| X-5P | 120 @30Hz | 5K (3.5K) | 4Hz~100kHz | 150 / 550 | 420 / 8 | 0.17 | 300B, EL-156, KT-88, 6550A, 350B, KT-66, EL-34, 6L6GC |
| X-3.5P | 120 @30Hz | 3.5K | 4Hz~100kHz | 120 / 420 | 500 / 12 | 0.16 | — |
| XE-65-1 | 65 @30Hz | 1K | 4Hz~80kHz | 30 / 60 | 800 / 25 | 0.19 | 6C33C |
| XE-60-8 | 60 | 8K | 4Hz~50kHz | 210 / 600 | 220 / 5 | 0.21 | KT-66, 6L6, KT-60T, 7591 |
| XE-60-6.6 | 60 | 6.6K | 4Hz~60kHz | 200 / 500 | 270 / 5 | 0.20 | — |
| XE-60-5 | 60 @30Hz | 5K | 4Hz~80kHz | 160 / 380 | 270 / 7 | 0.21 | 300B, EL-156, KT-88, 6550A, 350B, KT-66, EL-34, 6L6GC |
| XE-60-3.5 | 60 @30Hz | 3.5K | 4Hz~100kHz | 130 / 280 | 280 / 8 | 0.23 | — |
| XE-45-8 | 45 | 8K | 5Hz~50kHz | 230 / 550 | 200 / 5 | 0.24 | 6L6GC, 7591, CV-807, 6T6 |
| XE-45-5 | 45 | 5K | 5Hz~70kHz | 140 / 340 | 260 / 6 | 0.24 | 6CA7, 6L6GC, 2A3, 6B4G |

Notes:

• All models except XE-65 have SG (Screen Grid) tap.

• X-5P has a 1.4Ω secondary tap; with 8Ω load on this tap, primary impedance becomes ~5.5kΩ.

• XE-60 and XE-45 series include cathode NFB winding, usable for overall NFB.

• Case: X series = light gray leather-tone with cage-type R cover. XE-60/XE-45 = gray hammer-net finish.

## Single-Ended Types — Secondary: 4Ω, 8Ω, 16Ω

| Model | Power (W) | Primary Zp (Ω) | Response (-3dB) | Primary L (H) Min/Max | DC Current Max (mA) | Loss @16Ω (dB) | Recommended Tubes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X-10S | 40 | 10K | 20Hz~55kHz | 65 / 80 | 130 | 0.25 | VT-4C, UV-211, 845 |
| X-5S | 40 @30Hz | 5K | 18Hz~70kHz | 35 / 42 | 200 | 0.23 | PX-25, PX-25A, DA-30, 50 |
| X-3.5S | 40 @30Hz | 3.5K | 15Hz~80kHz | 28 / 38 | 250 | 0.22 | 300B, PX-25, 50 |
| X-2.7S | 40 @30Hz | 2.7K | 15Hz~80kHz | 25 / 30 | 300 | 0.20 | 300B, 2A3 |
| XE-60-3.5S | 30 | 3.5K | 20Hz~100kHz | 20 / 26 | 170 | 0.20 | 300B, 50, PX-25 |
| XE-60-2.5S | 30 | 2.5K | 20Hz~100kHz | 15 / 20 | 180 | 0.21 | 2A3, 300B |
| XE-60-10SNF | 40 @40Hz | 10K | 30Hz~50kHz | 40 / 45 | 90 | 0.24 | UV-211A, 845 |
| XE-60-5SNF | 40 @40Hz | 5K | 25Hz~70kHz | 24 / 26 | 140 | 0.21 | 50, PX-25 |
| XE-60-3SNF | 20 | 3K | 20Hz~80kHz | 18 / 23 | 170 | 0.22 | 300B, 2A3 |
| XE-20-600S | 20 | 600 | 30Hz~100kHz | 2.2 / 3.0 | 320 | 0.34 | 6C33C |

Notes:

• X-10S is a long-size model (180mm height) with balanced proportions.

• XE-60-S series has SG tap. XE-60-SNF has NFB winding.

• XE-20-600S secondary: 4Ω, 6Ω, 8Ω, 16Ω.

• Case: Gray hammer-net finish (X series has cage-type R cover).

## Single-Ended Universal Output Transformer (with SG Tap)

| Model | Power | Impedance Primary → Secondary | Response (-3dB) | Primary L (H) Min/Max | DC Current (mA) | Loss (dB) | Recommended Tubes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| XE-20S (20W @50Hz) | 20W | 5K → 8, 16 | 35Hz~70kHz | 16 / 18 | 160 | 0.26 | 300B, 50 |
|  |  | 3.5K → 8, 16 | 35Hz~70kHz |  |  | 0.33 | PX-25, DA-30 |
|  |  | 2.5K → 8, 16 | 25Hz~90kHz |  |  | 0.33 | VT-52, 2A3 |
|  | — | — | 20Hz~90kHz |  |  | 0.47 | 350B, KT-88, KT-66, 6550A |

• Case: Gray hammer-net finish.

Output transformers are critical components that determine the sound quality of tube amplifiers. The X and XE series are TANGO's flagship products, recommended with confidence as ultimate output transformers. They utilize the excellent magnetic properties of grain-oriented Orient Hi-B material in a cut-core construction, combined with high-quality insulation materials. Advanced know-how and modern technology deliver wide bandwidth, low distortion, and high-grade performance.

### Dimensions (mm) — X / XE Series (Enclosed Type)

| Model | A | B | Height | a | b | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- |
| X-10S | 125 | 125 | 180 | 80 | 80 | 7.7 |
| X-8P, X-5P, X-3.5P, X-5S | 125 | 125 | 138 | 80 | 80 | 6.4 |
| X-3.5S, X-2.7S | 125 | 125 | 138 | 80 | 80 | 6.4 |
| XE-20, XE-45 | 83 | 78 | 104 | 60 | 55 | 2.3 |
| XE-60 / XE-65 | 110 | 100 | 114 | 80 | 80 | 4.0 |

# OUTPUT TRANSFORMERS — Additional Series

## Single-Ended (Permalloy Core) — Secondary: 4Ω, 6Ω, 8Ω, 16Ω

| Model | Power (W) | Primary Zp (Ω) | Response (-3dB) | Primary L (H) Min/Max | DC Current (mA) | Loss (dB) | Recommended Tubes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NY-15-3.5S | 15 @30Hz | 3.5K (SG) | 20Hz~45kHz | 23 / 30 | 200 | 0.41 | 300B, PX-25, 50 |

• Dimensions same as XE-60. Gray hammer-net finish.

## Single-Ended (Orient Core) — Secondary: 4Ω, 8Ω, 16Ω

| Model | Power (W) | Primary Zp (Ω) | Response (-3dB) | Primary L (H) Min/Max | DC Current (mA) | Loss (dB) | Recommended Tubes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FW-150-10SR | 40 | 10K (SG) | 25Hz~35kHz | 40 / 53 | 130 | 0.27 | VT-4C, UV-211, 845 |
| FW-150-35SR | 40 @40Hz | 3.5K (SG) | 20Hz~80kHz | 23 / 45 | 250 | 0.23 | 300B, PX-25, EL-34 |
| FX-50-7S | 30 | 7K | 25Hz~70kHz | 30 / 50 | 130 | 0.22 | VT-62×5, UV-211, 845 |
| FX-50-5S | 30 | 5K | 25Hz~70kHz | 25 / 36 | 170 | 0.23 | DA-30, PX-25, PX-25A, 50 |
| FX-50-3.5S | 30 @40Hz | 3.5K | 25Hz~80kHz | 18 / 27 | 200 | 0.23 | 300B, PX-25, 50 |
| FX-50-2.5S | 30 | 2.5K | 20Hz~130kHz | 14 / 21 | 250 | 0.26 | 300B, 2A3 |
| FW-20-14S | 20 | 14K | 30Hz~35kHz | 40 / 80 | 65 | 0.28 | 801A, VT-62 |
| FW-20-7S | 20 | 7K | 30Hz~60kHz | 24 / 30 | 120 | 0.24 | VT-62×5, 6AQ5, 6F6 |
| FW-20-1.2S | 20 @60Hz | 1.2K | 30Hz~60kHz | 4.5 / 6 | 240 | 0.33 | 6C41C, E130L |
| H-5S | 10 @80Hz | 5K (SG) | 60Hz~80kHz | 11 / 17 | 65 | 0.40 | 7189, 6BQ5 |
| M-705 | 7 @80Hz | 5K (SG) | 45Hz~70kHz | 13 / 18 | 60 | 0.43 | 6BQ5, 6V6 |

• M-705 secondary: 4Ω, 6Ω, 8Ω. Dimensions same as NC-18 etc.

## Push-Pull (Orient Core) — Secondary: 4Ω, 8Ω, 16Ω

| Model | Power (W) | Primary Zp (Ω) | Response (-3dB) | Primary L (H) Min/Max | DC Current Total (mA) | Loss (dB) | Recommended Tubes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FW-100-5R | 100 @30Hz | 5K | 5Hz~50kHz | 170 / 680 | 350 | 0.30 | 300B, EL-156, KT-88, 6550A |
| FW-100-5 | 100 | 5K | 5Hz~50kHz | 170 / 680 | 350 | 0.30 | 250B, KT-66, EL-34, 6L6GC |
| FW-100-3.5R | 100 @30Hz | 3.5K | 5Hz~60kHz | 100 / 490 | 420 | 0.30 | — |
| FW-100-3.5 | 100 | 3.5K | 5Hz~60kHz | 100 / 490 | 420 | 0.30 | — |
| FX-50-16G | 50 @30Hz | 16K (SG×L) | 9Hz~50kHz | 550 / 2600 | 140 | 0.32 | VT-62, 801A |
| FW-50-5 | 50 | 5K | 5Hz~60kHz | 170 / 640 | 260 | 0.30 | KT-88, 6550A, EL-34, KT-66 |
| FW-50-3.5 | 50 @30Hz | 3.5K | 5Hz~80kHz | 100 / 440 | 330 | 0.30 | 6L6GC, 6B4G, 2A3, VT-52 |
| FX-40-10 | 40 @45Hz | 10K | 4Hz~40kHz | 300 / 900 | 180 | 0.27 | VT-62, VT-25, 6F6, 6V6 |
| FX-40-8 | 40 | 8K | 4Hz~50kHz | 280 / 580 | 200 | 0.24 | 6V6, 6L6GC, 6G4A, 6F6 |
| FX-40-5 | 40 | 5K | 4Hz~80kHz | 170 / 350 | 260 | 0.24 | 2A3, 6B4G, 6CA7, 6L6GC |
| CRD-8 | 25 @50Hz | 8K | 10Hz~60kHz | 85 / 210 | 160 | 0.30 | 7189A, 6BQ5, 6V6 |
| CRD-5 | 25 | 5K | 10Hz~100kHz | 50 / 130 | 200 | 0.30 | 6B4G, 2A3, 6BN8 |

• FX-50-16G has 4.5G 3×7P secondary tap. FW-100-5R/3.5R have cage-type R cover. FX-50-16G has light gray leather-tone case.

### Dimensions (mm) — Additional Series

| Model | A | B | Height | a | b | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- |
| FW-150 | 125 | 125 | 138 | 80 | 80 | 6.2 |
| FW-100 | 125 | 125 | 114 | 80 | 80 | 5.9 |
| FX-50-5S/3.5S, FX-50-7S/2.5S, FW-50 | 117 | 117 | 104 | 80 | 80 | 4.6~5.2 |
| FX-40, FW-20 | 83 | 78 | 104 | 60 | 55 | 2.3 |
| CRD | 83 | 75 | 80 | 60 | 55 | 1.6 |
| H-5S | 65 | 65 | 70 | 50 | 50 | 0.9 |

# OUTPUT TRANSFORMERS — Universal & Preamp Types

## Single-Ended Universal Output Transformers (SG Tap)

| Model | Power | Impedance Primary → Secondary | Response (-1dB) | Primary L (H) Min/Max | DC Current (mA) | Loss (dB) | Recommended Tubes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FW-20S | 20W @55Hz | 5K → 8 | 50Hz~70kHz | 12 / 18 | 160 | 0.34 | 350B, KT-88, KT-66, VT-52, 2A3, 6B4G |
|  | @45Hz | 3.5K → 8 | 35Hz~80kHz |  |  |  |  |
|  | @40Hz | 2.5K → 8 | 25Hz~90kHz |  |  |  |  |
| U-808 | 20W @50Hz | 5K → 8, 16 | 50Hz~50kHz | 11 (21) / 21 (25) | 130 | 0.50 | KT-88, 6550A, KT-66, 71A, 7591, 6L6, 2A3, EL34 |
|  | @40Hz | 3.5K → 8, 16 | 35Hz~60kHz |  |  |  |  |
|  | @35Hz | 2.5K → 4, 8 | 25Hz~65kHz |  |  |  |  |
| U-708 | 10W @30Hz | 2K → 4, 8 | 20Hz~65kHz | 14 / 18 | — | 0.80 | 6F6, 6AC5, 6BQ5, 6V6, 7591, EL34 |
|  | @70Hz / 60Hz | 7K / 5K → 6, 8 | 55Hz~45kHz / 40Hz~60kHz |  |  |  |  |
| U-608 | 6W @80Hz | 5K → 4, 8 | 80Hz~70kHz | 10 / 16 | 65 | 0.35 | 6F6, 6AC5, 6BQ5, 6V6, 6BM8, 6AR5 |
|  |  | 2.5K → 16 |  |  |  |  |  |

• U-608 has no SG tap. FW-20S and U-708 use Orient core. FW-20S dimensions on previous page.

## Push-Pull Types (Small) — Secondary: 4Ω, 8Ω, 16Ω

| Model | Power (W) | Primary (Ω) | Response (-1dB) | Primary L (H) Min/Max | DC Current (mA) | Loss (dB) | Recommended Tubes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| U15-8 | 15 @60Hz | 8K | 15Hz~100kHz | 50 / 230 | 160 | 0.30 | 7189A, 6BQ5, 6V6 |
| U15-5 | 15 | 5K | 15Hz~120kHz | 45 / 140 | 160 | 0.30 | 2A3, 6BM8, 6CA7(T) |
| U13-10 | 13 | 10K | 30Hz~50kHz | 60 / 270 | 100 | 0.36 | 6AR5, 6AQ5, 6BQ5(T) |
| U13-8 | 13 @60Hz | 8K | 20Hz~75kHz | 50 / 230 | 130 | 0.33 | 6BQ5, 6V6 |
| U13-5 | 13 | 5K | 20Hz~130kHz | 35 / 140 | 160 | 0.34 | 6BM8 |

Dimensions: W94(70) × D83(70) × H85(84) mm | Mounting: W68(52) × D65(56) mm | Weight: 1.8 (1.4) kg | Mounting screws: 4mm.

## Preamp Output Transformers (Magnetically Shielded)

| Model | Primary Z (Ω) | Secondary Z (Ω) | Turns Ratio (1st:2nd) | Response | DC Current (mA) | Max Output Voltage | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NP-406N (Push-Pull 1st) | 40K (R-P same) | 600 (CT) | 1:0.123 (Full 1st:Full 2nd) | 4Hz~30kHz @10kΩ 15Hz~35kHz @40kΩ (-1dB) | 40 (2nd side) Max 51 | 3Vrms @30Hz | Permalloy core Low distortion at normal levels |
| NP-216N (Push or Single) | 20K (5K Split) | 600 (150 Split) | 1:0.18 | 20Hz~30kHz (-2dB) @20kΩ, 5mA | 12 | 10Vrms @40Hz | Permalloy core |
| NP-206 (Single or Push) | 20K (5K Split) | 600 (150 Split) | 1:0.186 | 20Hz~30kHz (-2dB) @20kΩ, 10mA | 20 | 30Vrms @40Hz | Orient Hi-B cut core High output |
| NP-8 (Single or Push) | 10K (2.5K Split) | 600 (CT) | 1:0.245 | 5Hz~50kHz (-2dB) @10kΩ, 5mA | 20 | 30Vrms @30Hz | Orient Hi-B cut core High output |

• NP-406N & NP-216N: Permalloy core for low distortion at normal levels.
• NP-206 & NP-8: Orient Hi-B cut core for high output capability.
• Dimensions same as NC-18 etc. (see later pages).

# POWER TRANSFORMERS

## Square Case Types (MS-D / ME Series) — Magnetically Shielded

| Model | Primary | Secondary 1 (HT) | Secondary 2 | Secondary 3 (Filament) | Recommended Use |
| --- | --- | --- | --- | --- | --- |
| MS-505 | 100V | 0-360~380V 0.3A (Bridge) DC500mA | 0-75~150V 0.18A (Bridge) DC100mA | 6.3V 6.3V 6.3V 6.3V 4A each | KT-88, 6550A PP×2, KT-66, EL-156 |
| MS-450D / DR | 100V | 0-20-300~340V 0.85A (Bridge) DC450mA | 70V 60mA + 6.3V 4A | 6.3V 6.3V 4A each | KT-88, 6550A PP×2, KT-66, EL-156 |
| MS-380D / DR | 100V | 0-20-220-420~440V 0.7A (Bridge) DC380mA | 70V 60mA + 6.3V 4A | 6.3V 6.3V 4A each | KT-88, 6550A PP×2 |
| MS-330D | 100V | 0-10-145~165~185V 1.2A (Doubler) DC330mA | 60V 150mA + 6.3V 3.5A | 6.3V 6.3V 3.5A / 2A | KT-88 PP, 6CA7, 6L6GC PP×2 |
| MS-UVD | 100V | 0-370~400V 0.6A (Doubler) DC160mA | 150-0-150V 50mA + 10V 5A | 10V 10V 5A each | UV-211A, UV-845 SX2 |
| MS-210DR / LH | 100V | 370-0-370V 0.37A (Bridge) DC210mA | 150-0-150V 50mA + 10.5V 5A | 10.5V 10.5V 5A each | UV-211A |
| ME-275 | 100V | 320~280-0-179-280~320V AC 0-275V / DC CT 300mA Si 260mA | 5V 0.25A + 6.3V 5.2A / 3A | 0.25-6.3V 5.2A / 3A | 2A3, 6B4G (Lug terminals) |
| ME-225 | 100V | 400~360-0-100-360~400V AC 225mA / DC CT 250mA Si 210mA | 5V 0.5A + 6.3V 3.3A | 0.5-6.3V 3.3A each | 300B, KT-88, KT-66 PP Mono (Lug) |
| ME-211 | 100V | 370-0-370V 0.22A (Bridge) DC120mA | 150-0-150V 50mA + 10V 5A + 6.3V 2A / 3A | 10V / 6.3V 5A / 2A / 3A | UV-211A Single/Mono |

• MS-505, MS-450DR, MS-380DR, MS-210DR: R-type (rounded corners).
• MS-505 case: Light gray leather-tone to match PP output transformer series.
• MS-210DR-LH: Gray hammer-net to match X-10S single-ended output transformer.
• ME series: Same gray hammer-net finish as XE-60.

## Chassis-Mount / Flat Types (Lug) — Magnetically Shielded

| Model | Primary | Secondary 1 (HT) | Secondary 2 / 3 | Recommended Use |
| --- | --- | --- | --- | --- |
| MS-330 | 100V | 0-10-145~165~185V 1.2A (Doubler) DC330mA | 60V 150mA + 6.3V 3.5A | 6CA7 PP×2 |
| MS-400 | 100V | 0-115~125V AC1.4A (Doubler) DC400mA | 320V 100mA + 70V 100mA + 7V 7A + 6.3V 3A | 6C33C OTL-E (Lug) |
| MS-360 | 100V | 10-10-240~260~280V 0.57A (Bridge) DC300mA | 80V 60mA + 330V 100mA + 2.5V 5.2A | 2A3, 6B4G PP×2 |
| MS-250CT | 100V | 450~400-0-90-400~450V AC250mA / DC CT 220mA Si 180mA | 5V 4A + 5V 2A + 0.6-3~10V 4A | 300B SX2 |
| MS-200CT-A | 100V | 360~320-0-120-320~360V AC250mA / DC CT 200mA Si 180mA | 3V 3A + 0.4-5~0.6-3~10V 3A | 300B, PX-25 PP Mono / PP×4 |
| MS-160 | 100V | 320~280-0-70-280~320V AC150mA / DC CT 160mA Si 145mA | 5V 3A + 2.5V 3A | 2A3 PP / SX2 |
| MS-105 | 100V | 480~250-0-250~480V DC CT 110mA Si 100mA | 0.5-6.3V 3A + 7.5V 2A | 801A, VT-62 SX2 |
| MS-140 | 100V | 380~350~0-90~350~380V AC0-14A / DC 145mA Si 135mA | 0-5~7.5V 2.5A + 5V 3A + 6.3V 2A | VT-62, 300B SX2 |

• MS-330, 360, 250CT, 200CT-A available on request in square case to match output transformer color scheme.
• Orient core used in many models.

## Preamp Power Transformers (Lug / Terminal Board)

| Model | Primary | HT Secondary | Filament | Notes |
| --- | --- | --- | --- | --- |
| EV-3S | 100V | 260-0-260V AC35mA / DC Si 35mA | 15V 1.5A (DC 0.9A) + 0-5~6.3V 2A / 1A | SR shielded, case type, choke also usable |
| CV-2 | 100V | 260V AC30mA / Bridge DC20mA | 15V 1.5A (DC 0.9A) | Cut core, side-mount type |

# POWER TRANSFORMERS — Terminal Board & Lug Types

## Terminal Board Type (伏型)

| Model | Primary | HT Secondary | Filament 1 | Filament 2 / 3 | Recommended Use |
| --- | --- | --- | --- | --- | --- |
| ST-220 | 100V | 280~250-0-250~280V AC220mA / DC 220mA Si 210mA | 0-5~6.3V 3A | 6.3V 2.5A / 1.2A | 6BQ5, 6V6 PP×2 / 6B4G, VT-52 SX2 |
| LH-150 | 100V | 300~280-0-280~300V AC140mA / DC 150mA Si 140mA | 5V + 0-5V | 6.3V 3A | 6BQ5 PP / SX2, 6V6 |
| N-12 | 100V | 250~220-0-220~250V AC120mA / DC 120mA Si 120mA | 0-5~6.3V 2A | 6.3V 2.5A | 6BQ5 PP×2, 6BM8 |

## Lug Type (伏型) — Many with Magnetic Shielding (SR)

| Model | Primary | HT Secondary | Filament / Other | Notes / Use |
| --- | --- | --- | --- | --- |
| MX-520 | 100V | 0~10~160~180~200V AC0.95A (Bridge) DC520mA | 300V 100mA + 6.3V 6.5A | SR shielded, 6C33C SX2 |
| ST-350 | 100V | 0~175~350V 0.65A (Bridge) DC350mA | 70V 0.1A + 6.3V 5A | EL-34, 6L6GC, 350B PP×2 |
| MX-280 | 100V | 320~280-0-70-280~320V AC280mA / DC 300mA Si 260mA | 0~2.5~6.3~5V 5.2A | SR, 2A3/6B4G, 6L6, VT-52 |
| MX-175 | 100V | 450~400~360-0~360~400~450V AC/DC (Choke input 250mA / Condenser 170mA) | 5V 3A + 6.3V 3A | SR, 2A3/6B4G PP×2, KT88/KT66 PP |
| MX-165 | 100V | 530~400~250-0-250~400~530V AC150mA / DC 5R4G 170mA | 5V 3A + 7.5V 4.5A + 6.3V 3A | SR, VT-62, VT-25, 10 |
| MX-205 | 100V | 320~280-0~70-280~320V AC205mA / DC 210mA Si 200mA | 5V + 6.3V 3A / 2A | 6BQ5, 6V6 PP×2, 6F6, 6L6 |
| MX-270 | 100V | 180~160-0~160~180V AC270mA / DC 280mA Si 260mA | 5V + 6.3V 3A / 2A | SR, 6C41C / EL30L SX2 |
| ST-230 | 100V | 0~7.5~105~120~135V AC0.83A DC230mA | 36V 0.36A + 6.3V 2A | 6BQ5/6BM8 PP×2, 6L6GC SX2 |
| MX-135 | 100/105V | 280~250-0-250~280V AC0.135A / DC 140mA Si 130mA | 5V + 6.3V 2A | 6BQ5/6V6 PP / SX2 |
| PH-120 | 100V | 280~250-0-250~280V AC120mA / DC 120mA Si 110mA | 0-5~6.3V 2A / 1.2A / 3A | 6BQ5/6V6 PP, 6F6 |
| PH-100 / S | 100V | 280~250-0-250~280V AC100mA / DC Si 100mA | 0-5~6.3V 2A / 2.5A | PH-100: 7189A/6BQ5; PH-100S: SR preamp |
| PH-70 | 100V | 280~250-0-250~280V AC70mA / DC Si 70mA | 0-5~6.3V 2A / 3A | 6BQ5, 6AR5, 6V6, 6AQ5 |
| ST-55S | 100/110V | 0~20-280~340V 100mA (Bridge) DC55mA | 0-6.3~12.6V 2.5A DC1.5A + 0-3.15~6.3V | Preamp, SR shielded |
| ST-30S | 100V | 260-0-260V AC30mA / DC Si 30mA | 0-15~17V 2A (DC1.2A) + 6.3V 1A | Preamp, SR shielded |

• Many MX/ST/PH models available on request in square case matching output transformer colors.
• SR = Magnetically shielded (steel case).

### Dimensions (mm) — Power Transformers

| Model Group | A | B | C | Mounting | Chassis Hole | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- |
| MS-400 / MX-520 | 120 | 105 | 110 | 36×95 | 76×88 | 6.5 |
| MS-330/360/250CT/200CT-A / MX-280/175/165 | 120 | 105 | 95~100 | 32×95 | 76×88 | 5.3~5.7 |
| MS-160/105/140 / MX-205/270 | 110 | 96 | 84 | 30×87 | 70×78 | 3.8 |
| ST-350 | 115 | 100 | 100 | 28×95 | 76×85 | 5.2 |
| ST-230 / ST-220 | 107 | 93 | 84 | 28/35×87 | 70×78 | 3.6~3.7 |
| LH-150 | 99 | 83 | 80 | 35×82 | 73×58 | 2.9 |
| N-12 / PH-100 / PH-70 / MX-135 | 88~93 | 76~79 | 67~73 | 25~30×72 | 62×53~59 | 2.1~2.8 |
| ST-55S | 93 | 79 | 73 | 26×72 | 62×53 | 2.3 |
| ST-30S | 82~83 | 70 | 62~76 | 23~25×65 | 55~58×46~48 | 1.4~1.5 |

# CHOKES & COILS

## Square Case Chokes (Choke Input Rectification)

| Model | Rating | Safe Current | DCR @20°C | Max AC Input (EAC=1) | Ripple Current @1mm |
| --- | --- | --- | --- | --- | --- |
| CH-20-150D | 20H 150mA | 200mA | 210 Ω | 700V | 35mA |
| CH-12-200D | 12H 200mA | 260mA | 120 Ω | 600V | 50mA |
| CH-5-300D | 5H 300mA | 350mA | 45 Ω | 500V | 100mA |

• In choke input, higher AC voltage increases hum and leakage flux compared to capacitor input.
• Ripple current is proportional to input AC voltage; lower EAC reduces ripple.
• Standard: Black. On request: Light gray leather-tone (PP match) or gray hammer-net (SE match).
• Withstand voltage: AC 2kV 1 min.

## Filter Chokes (After Capacitor Input Rectification)

| Model | Rating | Safe Current | DCR | Notes |
| --- | --- | --- | --- | --- |
| MC-15-150D | 15H 150mA | 200mA | 155 Ω | □ High-grade square case |
| MC-10-200D | 10H 200mA | 260mA | 95 Ω | □ High-grade square case |
| MC-5-250D | 5H 250mA | 330mA | 67 Ω | □ High-grade square case |
| MC-3-350D | 3H 350mA | 450mA | 42 Ω | □ High-grade square case |
| MC-1.5-500D | 1.5H 500mA | 650mA | 20 Ω | □ High-grade square case |
| EC-20-120 | 20H 120mA | 140mA | 300 Ω | Iron cover, magnetically shielded |
| EC-10-180 | 10H 180mA | 200mA | 150 Ω | Iron cover, magnetically shielded |
| EC-4-280 | 4H 280mA | 300mA | 60 Ω | Iron cover, magnetically shielded |
| 0.1mH3.5 | 0.1mH 3.5A | 4A | 0.2 Ω | — |
| 0.1H1200 | 0.1H 1.2A | 1.5A | 1.9 Ω | — |
| 0.7H500 | 0.7H 500mA | 600mA | 10 Ω | — |
| 1H400 | 1.2H 400mA | 460mA | 17 Ω | — |
| 3H250 | 3H 250mA | 300mA | 44 Ω | — |
| 10H130 | 10H 130mA | 150mA | 165 Ω | — |
| 10H100 / C-110 | 10H 100mA | 120mA | 160~200 Ω | C series |
| C-112 / C-115 | 10H 120~150mA | 150~190mA | 165~135 Ω | C series |
| C-512 / C-515 / C-520 / C-525 | 5H 120~250mA | 140~290mA | 100~82 Ω | C series |

• □ = High-grade square case. ◎ = SR magnetically shielded.
• Low DC magnetization inductance drop. Iron cover provides magnetic shielding.
• Minimal long-term insulation degradation.
• Withstand voltage: AC 2kV 1 min.
• Dimensions for MC/EC: W80(70)×D80(70)×H93(84) mm, mounting W66(52)×D63(56) mm, weight 1.8(1.4) kg.

# DRIVER / INPUT TRANSFORMERS & SPECIAL CHOKES

## Driver/Input Transformers — Cut Core, Magnetically Shielded

(For strong drive of output tubes. DC superposition possible on primary.)

| Model | Primary Zp (Ω) | Turns Ratio (1st:2nd) | Response (-2dB @4V) | Max DC (mA) | Primary L (H @ DC) | Max Output Voltage | Driver Example |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NC-10 (2nd SE or PP) | 5K | 1:1+1 | 40Hz~30kHz (rp=5kΩ, Ib=10mA) | 15 | 30H @10mA | 200Vrms (G+G) @50Hz | 6350# (6SN7 class) |
| NC-14 (2nd SE or PP) | 5K | 1+1:1+1 | 25Hz~40kHz (rp=5kΩ, Ib=15mA) | 30 | 50H @15mA | 160Vrms (G+G) @40Hz | 6SN7 etc. |
| NO-15 (2nd SE) | 10K | 1:2 | 35Hz~35kHz (rp=10kΩ, Ib=5mA) | 15 | 60H @5mA | 150Vrms @50Hz | 6SN7 (rp=rs=12kΩ) |
| NC-16 (2nd SE or PP) | 7K | 1+1:2+2 | 25Hz~20kHz (rp=7kΩ, Ib=7mA) | 15 | 70H @7mA | 200Vrms (G+G) @40Hz | 6SN7 (rp=5~10kΩ) |
| NC-18 (2nd SE or PP) | 8K | 1+1:1.5+1.5 | 25Hz~25kHz (rp=8kΩ, Ib=10mA) | 15 | 80H @10mA | 200Vrms (G+G) @35Hz | 6SN7 (rp=5~10kΩ) |
| NO-20 (2nd SE) | 5K | 1:1 | 18Hz~80kHz (rp=5kΩ, Ib=20mA) | 30 | 60H @20mA | 150Vrms @10Hz | EL34(T) etc. (rp=1~10kΩ) |
| NC-19N (2nd SE or PP) | 10K | 1+1:1.5+1.5 | 35Hz~40kHz (rp=10kΩ, Ib=5mA) | 8 | 70H @5mA | 120Vrms (G+G) @50Hz | 6SN7 (rp=5~10kΩ) |

• NC-19N uses Permalloy core. NO-20 dimensions same as CH-D, weight 2.0 kg.

## Push-Pull Primary / Push-Pull Secondary Driver Transformers

| Model | Primary Zp (Ω) | Turns Ratio (1st:2nd) | Response (-1dB @4V) | Primary L (5V 50Hz) | Rated DC (mA) | Max Output @40Hz DC | Driver Example |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NC-22 | 5K | 1+1:1+1 | 5Hz~40kHz | 300H | 100 | 3 / 400Vrms | 6V6(T), 6F6(T), EL34(T) |
| NC-9 | 5K | 1+1:2+2 | 20Hz~25kHz | 140H | 80 | 2 / 250Vrms | 6V6(T), 6F6(T), EL34(T) |

• NC-22 dimensions same as CH-D, weight 1.8 kg.

## Line Input Transformers for Main Amplifier (Permalloy Core, Shielded)

Secondary: Single-Ended or Push-Pull

| Model | Primary Z (Ω) | Turns Ratio (1st:2nd) | Response (-1dB @0.3V) | Max Output @30Hz | Primary L (1V 50Hz) |
| --- | --- | --- | --- | --- | --- |
| NN-11 | 600 (CT) | 1:4+4 | 3Hz~30kHz | 60Vrms | 55H |
| NN-7 | 600 (CT) | 1:2+2 | 3Hz~30kHz | 30Vrms | 55H |
| NN-6 | 600 (CT) | 1:1+1 | 3Hz~120kHz | 15Vrms | 55H |

## Special Chokes (Cut Core / EL Core, Shielded)

| Model | Rating | Safe Current | DCR | Notes |
| --- | --- | --- | --- | --- |
| TC-30-50 | 30H 50mA | 80mA | 485 Ω | Cut core (TC series) |
| TC-160-15W | 160H 15mA (40H 30mA parallel) | 20mA (40mA) | 1160 Ω (290 Ω) | Cut core, plate choke usable |
| TC-60-35W | 60H 35mA (15H 70mA parallel) | 50mA (100mA) | 730 Ω (183 Ω) | Cut core, plate choke usable |
| TC-10-130W | 10H 130mA (2.5H 280mA parallel) | 150mA (300mA) | 130 Ω (33 Ω) | Cut core |
| TC-5-180W | 5H 180mA (1.3H 360mA parallel) | 200mA (400mA) | 76 Ω (19 Ω) | Cut core |
| SC-5-150 | 5H 150mA | 170mA | 85 Ω | EL core (SC series) |
| SC-3-200 | 3H 200mA | 220mA | 50 Ω | EL core |

• ( ) values when windings connected in parallel.
• TC-30-50, TC-160-15W, TC-60-35W usable as plate chokes.

株式会社 平田電機製作所

(Hirata Electric Manufacturing Co., Ltd.)

Head Office: 1-24-7 Nishiochiai, Arakawa-ku, Tokyo 116-0011, Japan

TEL: +81-3-3800-2251 | Factory: Kyoto / Maebashi

Note: Specifications, dimensions, and ratings are subject to change without notice for product improvement. Mounting dimensions do not include mounting screws. Original Japanese catalog: August 1996